# Solar Schools Programme: Maintenance Guide

This guide tells school staff how to keep a rooftop solar installation working. Read it before the first cleaning.

# 1. Monthly tasks

Do these tasks once a month, on a dry day:

Clean every panel with water and a soft cloth

Check that no cable is loose or exposed

Write the inverter reading in the log book

## 1.1 Cleaning steps

Switch off the inverter

Rinse the panels from the top down

Switch the inverter on and check the green light

# 2. Who does what

Table 1: Responsibilities

| Task | Person | How often |
| --- | --- | --- |
| Cleaning | Caretaker | Monthly |
| Inverter check | District engineer | Quarterly |
| Log review | Head teacher | Monthly |

The next table shows the fault codes; the first column is shared by two rows.

| Code | Meaning | Action |
| --- | --- | --- |
| E1 | Low voltage | Wait for sunlight |
|  | No voltage | Call the engineer |

Table 2: Fault codes

# 3. A clean installation

Figure 1: Panels after cleaning

Panels should look like this after every cleaning. Dust cuts output by a fifth.

A clean panel is a working panel.